# Evaporator Operations SOP

Infant Food Manufacturing Plant – IoT Monitoring Simulation
Equipment: Multi-Effect Evaporator System
Machine Type: evaporator
Monitoring Frequency: Every 5 seconds via IoT telemetry

## 1. Purpose

This SOP defines operational response procedures for abnormal evaporator conditions detected through the IoT monitoring system.

## 2. Process Overview

The evaporator removes excess water from liquid food slurry before spray drying and helps achieve the required solids concentration while preserving nutritional quality.

## 3. Critical Role in Infant Food Manufacturing

Improper evaporator operation may lead to burnt product, nutrient degradation, fouling, energy loss, batch rejection, and production downtime.

## 4. IoT Parameters Monitored

Steam Temperature: 85°C – 105°C | Abnormal: > 110°C
Pressure: 1.8 – 2.8 bar | Abnormal: > 3.0 bar
Liquid Level: 40% – 85% | Abnormal: < 30%

## 5. Sensor Definitions

Steam Temperature controls evaporation rate and protects nutrients.
Pressure maintains stable boiling conditions.
Liquid Level prevents dry heating and tube damage.

## 6. Alarm Severity Classification

Advisory → Trending toward limit
Warning → Near threshold
Critical → Threshold exceeded
Emergency → Shutdown required

## 7. SOP – High Steam Temperature

Condition: Steam Temperature > 110°C

Immediate Actions:
- Acknowledge alarm
- Verify sensor reading
- Reduce steam input
- Check feed flow
- Inspect condensate removal

Emergency Shutdown if temperature exceeds 125°C.

## 8. SOP – High Pressure

Condition: Pressure > 3.0 bar

Immediate Actions:
- Reduce steam supply
- Inspect vapor outlet
- Verify vacuum system
- Check safety relief devices

Emergency Shutdown if pressure exceeds 4.0 bar.

## 9. SOP – Low Liquid Level

Condition: Liquid Level < 30%

Immediate Actions:
- Verify feed system
- Reduce steam heating
- Inspect for leaks
- Confirm sensor validity

Emergency Shutdown if level drops below 15%.

## 10. Controlled Shutdown Procedure

1. Stop product feed gradually
2. Reduce steam supply slowly
3. Normalize pressure
4. Drain product safely
5. Flush evaporator
6. Isolate steam supply

## 11. Product Quality Protection Procedure

If abnormal conditions persist, place affected batch on HOLD and notify QA department.

## 12. Maintenance Inspection Checklist

Inspect steam valves, sensors, feed pumps, vacuum system, condensers, relief valves, and tube fouling condition.

## 13. Operator Logging Requirements

Operators must document alarm type, timestamp, readings, corrective actions, escalation details, and shutdown actions.

## 14. Escalation Matrix

Warning Alarm → Shift Operator
Critical Alarm → Shift Supervisor
Repeated Alarm → Maintenance Team
Quality Risk → QA Department
Emergency Shutdown → Plant Manager + Safety Team

## 15. Example IoT Alert Payloads

{
  "deviceId": "evaporator-01",
  "alert": true,
  "steamTemperature": 121.4
}

{
  "deviceId": "evaporator-01",
  "alert": true,
  "pressure": 3.8
}

{
  "deviceId": "evaporator-01",
  "alert": true,
  "liquidLevel": 18.5
}

## 16. Recommended Dashboard Features

Real-time charts, alarm banners, SOP guidance popups, maintenance ticket creation, QA hold simulation, and historical trends.

## 17. Simulation Notes

Abnormal events occur every 3–10 minutes. The simulation demonstrates realistic industrial monitoring workflows.