# Pasteurizer Operations SOP

Infant Food Manufacturing Plant – IoT Monitoring Simulation
Equipment: Continuous HTST Pasteurizer System
Machine Type: pasteurizer
Monitoring Frequency: Every 5 seconds via IoT telemetry

## 1. Purpose

This SOP defines the operational response procedures for abnormal pasteurizer conditions detected through the IoT monitoring system.

## 2. Process Overview

The pasteurizer thermally processes liquid infant food mixtures to ensure microbial safety and regulatory compliance before downstream processing.

## 3. Critical Importance in Infant Food Manufacturing

Improper pasteurization may lead to microbial contamination, product recalls, nutritional degradation, and regulatory non-compliance.

## 4. IoT Parameters Monitored

Temperature: 68°C – 82°C | Abnormal: < 65°C or > 85°C
Flow Rate: 150 – 450 L/hr | Abnormal: > 500 L/hr
Holding Time: 15 – 25 sec | Abnormal: < 10 sec

## 5. Parameter Definitions

Temperature ensures microbial destruction.
Flow Rate affects residence time.
Holding Time ensures validated pathogen reduction.

## 6. Alarm Severity Levels

Advisory → Drifting parameter
Warning → Near threshold
Critical → Threshold exceeded
Emergency → Food safety risk

## 7. SOP – Low Temperature

Condition: Temperature < 65°C

Immediate Actions:
- Acknowledge alarm
- Divert product to rework tank
- Verify temperature readings
- Check steam/heating system
- Reduce product flow

Emergency Shutdown if temperature falls below 55°C.

## 8. SOP – High Temperature

Condition: Temperature > 85°C

Immediate Actions:
- Reduce heating input
- Verify sensors
- Inspect flow stability
- Notify QA

Shutdown if temperature exceeds 95°C.

## 9. SOP – Low Holding Time

Condition: Holding Time < 10 sec

Immediate Actions:
- Divert product immediately
- Reduce flow rate
- Verify holding tube integrity
- Check controller settings

Shutdown if holding time drops below 5 sec.

## 10. SOP – High Flow Rate

Condition: Flow Rate > 500 L/hr

Immediate Actions:
- Reduce pump speed
- Verify flow meter
- Inspect control loop

Shutdown if flow exceeds 650 L/hr.

## 11. Product Diversion Procedure

Whenever CCP conditions are violated, divert product, place batch on HOLD, notify QA, and document the deviation.

## 12. Controlled Shutdown Procedure

1. Stop product feed gradually
2. Divert remaining product
3. Reduce steam supply
4. Flush system
5. Stop pumps
6. Record shutdown reason

## 13. QA & Regulatory Requirements

All CCP deviations require QA review, batch traceability, corrective action documentation, and supervisor approval.

## 14. Maintenance Inspection Checklist

Inspect sensors, steam valves, heat exchangers, flow meters, holding tubes, diversion valves, and PLC logic.

## 15. Operator Logging Requirements

Operators must record alarm type, timestamp, sensor values, batch number, corrective actions, and QA notifications.

## 16. Escalation Matrix

Warning Alarm → Shift Operator
CCP Deviation → QA Department
Repeated Alarm → Maintenance Team
Emergency Shutdown → Plant Manager + Safety Team

## 17. Example IoT Alert Payloads

{
  "deviceId": "pasteurizer-01",
  "alert": true,
  "temperature": 58.4
}

{
  "deviceId": "pasteurizer-01",
  "alert": true,
  "temperature": 92.7
}

{
  "deviceId": "pasteurizer-01",
  "alert": true,
  "holdingTime": 7.2
}

{
  "deviceId": "pasteurizer-01",
  "alert": true,
  "flowRate": 612.5
}

## 18. Recommended Dashboard Features

Real-time graphs, CCP indicators, alarm banners, SOP guidance, QA hold simulation, batch traceability, and deviation reports.

## 19. Simulation Notes

Abnormal events occur every 3–10 minutes with automatic alert triggering based on threshold conditions.