# Spray Dryer Operations SOP

Infant Food Manufacturing Plant – IoT Monitoring Simulation
Equipment: Industrial Spray Dryer System
Machine Type: spray_dryer
Monitoring Frequency: Every 5 seconds via IoT telemetry

## 1. Purpose

This SOP defines operational response procedures for abnormal spray dryer conditions detected through the IoT monitoring system.

## 2. Process Overview

The spray dryer converts concentrated liquid infant food slurry into dry powder using heated air and atomization technology.

## 3. Importance in Infant Food Manufacturing

Improper spray drying may lead to excess moisture, nutrient degradation, powder lumping, microbial risks, and fire hazards.

## 4. IoT Parameters Monitored

Inlet Temperature: 160°C – 195°C | Abnormal: > 200°C
Outlet Temperature: 85°C – 110°C | Abnormal: < 80°C or > 120°C
Humidity: 6% – 14% | Abnormal: > 15%

## 5. Parameter Definitions

Inlet Temperature controls drying energy.
Outlet Temperature indicates drying efficiency.
Humidity indicates final powder moisture conditions.

## 6. Alarm Severity Levels

Advisory → Trending abnormal
Warning → Near threshold
Critical → Threshold exceeded
Emergency → Unsafe condition

## 7. SOP – High Inlet Temperature

Condition: Inlet Temperature > 200°C

Immediate Actions:
- Reduce burner firing rate
- Inspect airflow system
- Verify temperature sensors
- Check product feed stability

Emergency shutdown if temperature exceeds 230°C.

## 8. SOP – High Humidity

Condition: Humidity > 15%

Immediate Actions:
- Verify heating performance
- Reduce slurry feed rate
- Inspect atomizer and exhaust airflow

Shutdown if humidity exceeds 25%.

## 9. SOP – High Outlet Temperature

Condition: Outlet Temperature > 120°C

Immediate Actions:
- Reduce inlet heat
- Increase feed rate slightly
- Verify airflow stability

Shutdown if outlet temperature exceeds 140°C.

## 10. SOP – Low Outlet Temperature

Condition: Outlet Temperature < 80°C

Immediate Actions:
- Increase drying temperature
- Reduce feed rate
- Inspect atomizer and exhaust system

Shutdown if outlet temperature falls below 60°C.

## 11. Powder Quality Protection Procedure

Hold affected powder lots, notify QA, perform moisture analysis, and inspect powder appearance before release.

## 12. Fire & Explosion Safety Procedure

If smoke or fire is detected, stop product feed, shut down burner, activate fire suppression system, and evacuate non-essential personnel.

## 13. Controlled Shutdown Procedure

1. Stop slurry feed gradually
2. Maintain airflow during cooldown
3. Reduce burner firing slowly
4. Allow chamber cooling
5. Clean powder collection system

## 14. Maintenance Inspection Checklist

Inspect burners, thermocouples, atomizers, blowers, cyclone separators, humidity sensors, and fire suppression systems.

## 15. Operator Logging Requirements

Operators must record alarm type, timestamp, sensor values, batch number, corrective actions, QA notifications, and shutdown actions.

## 16. Escalation Matrix

Warning Alarm → Shift Operator
Critical Alarm → Shift Supervisor
Powder Quality Risk → QA Department
Fire/Explosion Risk → Safety Team + Plant Manager

## 17. Example IoT Alert Payloads

{
  "deviceId": "spray-dryer-01",
  "alert": true,
  "inletTemperature": 221.4
}

{
  "deviceId": "spray-dryer-01",
  "alert": true,
  "humidity": 24.7
}

{
  "deviceId": "spray-dryer-01",
  "alert": true,
  "outletTemperature": 134.5
}

{
  "deviceId": "spray-dryer-01",
  "alert": true,
  "outletTemperature": 68.2
}

## 18. Recommended Dashboard Features

Real-time thermal graphs, humidity monitoring, alarm acknowledgment, SOP guidance, QA workflows, maintenance tickets, and fire safety alerts.

## 19. Simulation Notes

Abnormal conditions occur randomly every 3–10 minutes with automatic alert generation based on defined threshold limits.